Project:“Improving Solid Waste Management in Urban and Rural Areas of Bajina Basta, Visegrad and Han Pijesak municipalities “

Contract title - Purchasing special vehicles and equipment
Publication reference:48-00-00046/2016-28-8/T01

Questions and Answers
Contracting authority: Public Utility Company 12 Septembar
	Question
	Answer

	It is unclear that you require a vehicle with a total mass of 7500kg and the capacity of the drive axle itself is required min 6000kg. The load capacity of the required axle is also questioned regarding the required tire dimensions. Will you accept the drive axle of technical capacity min 5000kg? 
	Contracting authority will not accept permissible rear axle load less then required one by tender's technical specification, due to heavy duty working conditions and possibility of overloading the rear axle, bearing in mind the specific operating conditions of this type of vehicle/superstructure. The contracting authority remains on minimum requirements from the tender documentation.

	Will you accept engine with capacity max. 4500cm3? 


	The Contracting Authority has determined the maximum engine displacement based on the principles of economy and efficiency with regard to minimum fuel consumption, minimum motor oil used, lower maintenance expenses and environmental protection. The contracting authority remains on requirements from the tender documentation

	Will you accept engine with or without exhaust gas recirculation (EGR)? 


	As the vehicle will be intensively used in urban environments where high reliability and efficiency are required, the type of engine which has been required does not need periodic stops of the vehicle and special driver action to perform exhaust gas regeneration (there is no active DPF regeneration), has higher reliability  and has lower specific fuel consuption (g/kWh). Contracting Authority remains at the required technical specification in the tender documentation for engine without EGR

	Will you accept engine with capacity min. 10.500 cm3- max. 11.500 cm3? 


	In terms of technical specifications, the contracting authority determined the technical specifications of the vehicle and therefore defined the engine capacity according to its needs, guided by the principles of economy and efficiency. The required characteristics completely satisfy the needs of the client, they are not fixed and they are defined in the range as minimum and maximum acceptable. The contracting authority remains on requirements from the tender documentation.

	 According to the current situation in Serbia, and announced state of emergency, can you please inform us how you plan to proccede with the tender for Purchasing special vehicles and equipment for the project 48-00-00046/2016-28-8/T01? Do you plan to extend the deadline for submission bids?
	Please see the published corrigendum.

	Why the deadline for delivery of the trucks is 75 days bearing in mind that these are special vehicles
	The Contracting Authority has set a delivery period of 75 days for reasons of efficiency in the implementation of the project as well as the right to exercise the basic interest for the proper implementation of the project.

	Why is preferred one supplier of vehicles
	The Contracting Authority has determined the requirements based on the principles of economy and efficiency lower maintenance expenses and environmental protection in accordance with previously conducted Market research. The contracting authority remains on requirements from the tender documentation


PAGE  
2/2

